January 2014

DINION IP Bullet 5000 HD

  - SYSTEM DESCRIPTION

    - Video Surveillance Remote Devices

      - NTI-50022-V3 IP bullet 5000 HD camera

    - Performance Requirements

      - Complete network video surveillance system inside a bullet camera.

      - Built-in active infrared illuminator.

      - 1080p resolution.

      - Intelligent Dynamic Noise Reduction (iDNR) technology.

      - Varifocal 3 to 10 mm, DC Iris board-mounted lens.

      - Electronic day/night function.

      - Quad-streaming: Simultaneous dual H.264 streams, an M-JPEG stream and an I-frame stream.

      - Micro SDHC/SDXC card slot for edge recording up to 2 TB.

      - ONVIF conformant.

      - +12 VDC or Power over Ethernet (IEEE 802.3af compliant).

  - IP bullet 5000 HD [NTI-50022-V3]

    - General Characteristics:

      - The camera shall offer 1080p resolution at 16:9 aspect ratio.

      - The camera shall offer tamper and intelligent motion detection.

      - The camera shall offer two-way full duplex audio communication.

      - The camera shall have a built-in active infrared illuminator

      - The camera shall utilize Intelligent Dynamic Noise Reduction (iDNR) technology to reduce the bitrate and storage requirements by removing noise artifacts.

      - The camera shall accept power via Power over Ethernet (IEEE 802.3af compliant).

      - The camera shall be easy to install.

      - The camera shall provide four independent, fully programmable privacy masks.

      - The camera shall provide a user-defined Region-of-Interest (ROI) with electronic pan, tilt and zoom controls.

    - Imaging Requirements

      - The camera shall offer a 1/2.7-inch CMOS image sensor.

      - The camera shall offer a video resolution of 1080p @30 fps.

      - The camera shall offer 1920 x 1080 sensor pixels.

      - The camera shall be fitted with a board-mounted fixed varifocal 3 to 10 mm, DC Iris, F1.3 - 360 lens with a lens view angle of:

        - 36° - 107° (Horizontal)

        - 20° - 57° (Vertical).

      - The camera shall have a wide dynamic range of 76 dB.

      - The camera shall offer a minimum sensitivity of 0.24 lx (color) or 0.0lx (IR active).

      - The camera automatic electronic shutter shall offer a shutter speed of 1/30 (1/25) to 1/15000.

      - The camera shall be capable of capturing and storing images using the following compression standards:

        - H.264 MP (Main Profile)

        - M-JPEG

    - Network Video

      - The camera shall provide direct network connection using H.264 and M-JPEG compression and bandwidth throttling to efficiently manage bandwidth and storage requirements while delivering outstanding image quality.

      - The camera shall allow full camera control and configuration capabilities over the network.

      - The camera shall deliver HD-quality video, at rates up to 30 images per second, via TCP/IP over a 10/100 Base-T, auto-sensing, half/full duplex, RJ45 Ethernet connection.

      - The camera shall conform to the ONVIF Profile S standard.

      - The camera shall conform to protocols: IPv4, IPv6, UDP, TCP, HTTP, HTTPS, RTP/RTCP, IGMP V2/V3, ICMP, ICMPv6, RTSP, FTP, Telnet, ARP, DHCP, SNTP, SNMP (V1, MIB-II), 802.1x, DNS, DNSv6, DDNS, SMTP, iSCSI, UPnP (SSDP), DiffServ (QoS), LLDP, SOAP, Dropbox, CHAP, Digest Authentication.

    - Audio

      - The camera shall have line in and line out jack connectors for two-way, full duplex audio communication with audio compression AAC, G.711, L16 (live and recording).

    - Night Vision (Infrared)

      - The camera shall have a 12 LED high efficiency array (850 nm) for effective night vision up to 25 M (82 ft).

    - Control

      - The camera shall be configured via a web browser or PC surveillance software.

    - Connectors

      - The camera shall have one RJ-45 for Ethernet.

      - The camera shall have one analog video out BNC connector for installation only.

      - The camera shall have short or DC 5V activation alarm input.

      - The camera shall have one relay out (input rating maximum 1 A, 24 VAC/VDC).

      - The camera shall have one line in jack connector for audio input.

      - The camera shall have one line out jack connector for audio output

    - Electrical

      - The camera shall accept either +12 VDC or Power over Ethernet (48 VDC nominal).

      - The camera shall conform to the IEEE 802.3af (802.3at Type 1) compliant Power over Ethernet network. Power level: Class 3.

      - The camera shall consume 9.2 W (max.).

    - Environmental

      - The camera shall operate in a -30°C to +50°C (-22°F to +122°F) temperature range.

      - The camera can be stored in a -30°C to +70°C (-22°F to +158°F) temperature range.

      - The camera can be operated/stored in 20% to 90% relative humidity (non-condensing).

      - The camera shall offer IP66 ingress protection.

      - The camera shall offer IK08 impact protection.

    - Mechanical

      - The camera dimensions shall be (H x W x D): 103 x 100 x 252 mm (4.06 x 3.94 x 9.92 in

      - The camera weight shall be 1.6 kg (3.53 lb) approx.

      - The camera shall have 3-axis adjustment (350°/100°/350°).

      - The camera color is Pantone 877c.