May 2014

| Americas Bosch Security Systems, Inc. 130 Perinton Parkway Fairport, New York, 14450, USA Phone: + 1 800 289 0096 Fax: +1 585 223 9180 security.sales@us.bosch.com www.boschsecurity.us | Europe, Middle East, Africa Bosch Security Systems B.V. P.O. Box 80002 5617BAEindhoven, The Netherlands Phone: + 31 40 2577 284 Fax: +31 40 2577 330 emea.securitysystems@bosch.com www.boschsecurity.com | Asia-Pacific Robert Bosch (SEA) Pte Ltd, Security Systems 11 Bishan Street 21 Singapore 573943 Phone: +65 6571 2808 Fax: +65 6571 2699 apr.securitysystems@bosch.com www.boschsecurity.com |
| --- | --- | --- |

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2004 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

SECTION 28 23 29

VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

Bosch DINION IP starlight 8000 MP Camera

- – GENERAL

  - SUMMARY

    - Section Includes

      - Video Surveillance Remote Devices.

    - Related Sections

      - Section [28 23 13 – Video Surveillance Control and Management Systems].

      - Section [28 23 16 – Video Surveillance Monitoring and Supervisory Interfaces].

      - Section [28 23 19 – Digital Video Recorders and Analog Recording Devices].

      - Section [28 23 23 – Video Surveillance Systems Infrastructure].

**********Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

| Standards |  |
| --- | --- |
| Emission | EN 55022 Class B (2010), +AC (2011) FCC: 47 CFR 15, class B (2012-10-1) |
| Immunity | EN 50130-4 (PoE, +12 VDC)* (2011) EN 50121-4 (2006), +AC: (2008) |
| Alarm | EN 50130-5 Class II (2011) |
| Safety | EN 60950-1 UL 60950-1 (2nd edition) CAN/CSA-C 22.2 No. 60950-1 |
| Vibration | Camera with 500 g (1.1 lb) lens as per IEC 60068-2-6 (5 m/s2, operational) |
| HD | SMPTE 296M-2001 (Resolution: 1280x720) SMPTE 274M-2008 (Resolution: 1920x1080) |
| Color representation | ITU-R BT.709 |
| ONVIF conformance | EN 50132-5-2; IEC 62676-2-3 |

* Chapters 7 and 8 (mains voltage supply requirement) are not applicable to the camera. However, if the system in which this camera is used needs to comply with this standard, then any power supplies used must comply with this standard.

  - SYSTEM DESCRIPTION

    - Video Surveillance Remote Devices

      - NBN-80052-BA DINION IP starlight 8000 MP

    - Performance Requirements

      - Outstanding wide dynamic range (97 + 16 dB).

      - Superior 5 MP image capture at 30 fps, both in 4:3 and 16:9 aspect ratio.

      - Intelligent dynamic noise reduction reduces bitrate by up to 50% (giving reduced bandwidth and storage requirements).

      - Unprecedented low-light performance for 5MP camera (0.0121 lx).

      - Intelligent Video Analysis (IVA).

      - ONVIF conformant.

  - SUBMITTALS

    - Submit under provisions of Section [01 33 00].

    - Product Data:

      - Manufacturer’s data, user and installation manuals for all equipment and software programs including computer equipment and other equipment required for complete video management system.

    - Shop Drawings; include

      - System device locations on architectural floor plans.

      - Full Schematic of system, including wiring information for all devices.

    - Closeout Submittals

      - User manual.

      - Parts list.

      - System device locations on architectural floor plans.

      - Wiring and connection diagram.

      - Maintenance requirements.

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [10] years experience in manufacture and design Video Surveillance Devices.

      - Manufacturer’s quality system: Registered to ISO 9001 Quality Standard.

    - Video Surveillance System

      - Listed by [UL] [EN] [FCC] specifically for the required loads.  Provide evidence of compliance upon request.

    - Installer:

      - Minimum of [5] years experience installing Video IP Surveillance System.

  - DELIVERY, STORAGE AND HANDLING

    - Comply with requirements of Section [01 60 00].

    - Deliver materials in manufacture’s original, unopened, undamaged containers; and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

  - WARRANTY

    - Provide manufacturer’s warranty covering [3] years for replacement and repair of defective equipment.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users (up to 5 years after the EOL announcement).

    - Provide factory direct technical support from 8:00 a.m. to 8:00 p.m. via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:

[Bosch Security Systems, Inc.

130 Perinton Parkway

Fairport, New York, 14450, USA

Phone: + 1 800 289 0096

Fax: + 1 585 223 9180

security.sales@us.bosch.com

www.boschsecurity.us]

[Bosch Security Systems B.V.

P.O. Box 80002

5617 BA Eindhoven, The Netherlands

Phone: + 31 40 2577 284

Fax: +31 40 2577 330

emea.securitysystems@bosch.com

www.boschsecurity.com]

[Bosch Security Systems Pte Ltd

Robert Bosch (SEA) Pte Ltd, Security Systems

11 Bishan Street 21

Singapore 573943

Phone: +65 6571 2808

Fax: +65 6571 2699

apr.securitysystems@bosch.com

www.boschsecurity.com]

    - Substitutions: [Not permitted.] [Under provisions of Division 1.]

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

**********Specifier’s note: Select Camera System Series based on project requirement.

  - DINION IP starlight 8000 MP

[NBN-80052-BA]

    - General Characteristics:

      - The camera shall be a true day/night camera with a mechanical filter for truly outstanding nighttime performance. The filter can be switched remotely, or automatically via a light level sensor or contact input.

      - The camera shall utilize Intelligent Dynamic Noise Reduction (iDNR) technology to reduce the bitrate and storage requirements by removing noise artifacts.

      - The camera shall offer 5MP resolution.

      - The camera shall accept power via Power over Ethernet (IEEE 802.3af compliant).

      - The camera shall offer Intelligent Video Analysis (IVA).

      - The camera shall provide four independent, fully programmable privacy mask areas.

      - The camera shall provide an on-screen display to simplify the camera/lens back focus and network configuration settings.

      - The camera shall utilize pixel-by-pixel analysis to automatically compensate for bright areas of a high contrast scene (Back light) without having to define a window or area.

      - The camera shall provide intelligent Auto Exposure (iAE) to improve visibility of high contrast scenes (dark objects against a light background and the reverse).

      - The camera shall be easy to install.

      - The camera shall have nine configurable user modes with the best settings for a variety of applications.

    - Image processing and compression

      - The camera shall offer a 1/1.8-inch CMOS image sensor.

      - The camera shall offer 6.1MP sensor pixels.

      - The camera shall operate in low-light environments.

      - The camera shall use intelligent Dynamic Noise Reduction (iDNR) to actively analyze the contents of a scene and reduce noise artifacts accordingly.

      - The low-noise image and efficient H.264 compression technology shall provide clear images while reducing bandwidth and storage by up to 30% compared to other H.264 cameras.

      - The camera shall have a dynamic range of 97 dB WDR in 5MP mode (97+16 dB with iAE).

      - The camera shall provide the most usable image possible by cleverly optimizing the detail-to-bandwidth ratio.

      - The camera shall provide intelligent Auto Exposure (iAE) for superb front light compensation and incredible backlight compensation by automatically adapting to changing light conditions.

      - The camera shall provide user-defined Regions of Interest (ROI) where the remote E-PTZ (Electronic Pan, Tilt and Zoom) controls allow a user to select specific areas of the parent image.

    - Network Video

      - The camera shall provide direct network connection.

      - The camera shall allow full camera control and configuration capabilities over the network.

      - The camera shall be capable of capturing and storing images using the following compression standards:

        - H.264 MP (Main Profile)

        - M-JPEG

      - The camera shall deliver video, at rates up to 30 images per second, over a 10/100 Base-T, auto-sensing, half/full duplex, RJ45 Ethernet connection.

      - The camera shall comply with the IEEE 802.3af Power over Ethernet standard.

      - The camera shall conform to the ONVIF standard.

    - Intelligent Video Analysis (IVA)

      - The camera shall be capable of processing and analyzing video within the camera itself, with no extra hardware required.

      - The camera shall be capable of detecting and sending alarms for abnormal events.

      - The camera shall allow users to set up separate profiles, and switch profiles based on day/night or holiday schedules.

    - Surveillance Software

      - The camera shall be accessible from a web browser, with the Bosch Video Management System, with the free-of-charge Bosch Video Client, or via third-party software.

      - The camera shall be accessible from the Bosch Security System mobile app.

    - Access Security

      - The camera shall offer three levels of password protection.

      - The camera shall support 802.1x authentication using a RADIUS (Remote Authentication Dial In User Service) server.

      - The camera shall store a SSL certificate for use with HTTPS.

      - The camera shall be capable of being independently AES encrypted with 128-bit keys.

    - Image Posting

- The camera shall offer periodic JPEG image posting to an FTP server or a Dropbox account.

- The camera shall offer best face detection and JPEG best face image posting to an FTP server or to a Dropbox account.

    - Storage Management

      - The camera shall support iSCSI devices to allow video stream to be recorded directly to an iSCSI RAID array.

      - The camera shall support iSCSI storage targets to enable the camera to function as a conventional DVR.

      - The camera shall have a microSD card slot that uses standard; off-the-shelf microSD (SDHC and SDXC) cards for local storage (up to 2 TB).

      - The local storage feature shall be capable of storage for Automatic Network Replenishment (ANR).

    - Alarm Handling Features:

      - The camera shall provide an alarm input that may be triggered by either a normally opened or normally closed contact.

      - The camera shall provide the capability on alarm to display up to a 31 character, programmable alarm message.

      - The camera shall provide a relay output that may be selected for normally opened or normally closed operation. The relay can be activated from an external alarm input to the camera, manual activation from the browser, upon video motion detection, an alarm task script or video loss.

      - The camera shall provide email alarm messaging with optional JPEG posting.

    - Electrical

      - The camera shall accept either +12 VDC or Power over Ethernet.

      - The camera shall consume 9 W (max.).

    - Audio

      - The camera shall offer one (1) line in jack connector and one (1) line out jack connector.

      - The camera shall offer two-way, full duplex audio communication.

      - The camera shall offer G.711, AAC and L16 audio compression (live and recording).

    - Environmental

      - The camera shall operate in -20°C to +50°C (-4°F to +122°F) temperature range.

    - Technical Specifications:

| Power |  |
| --- | --- |
| Power Supply | 12 VDC Power-over-Ethernet 48 VDC nominal |
| Current Consumption | 500 mA (12 VDC) 175 mA (PoE 48 VDC) |
| Power Consumption | 9 W |
| PoE | IEEE 802.3af (802.3at Type 1) Class 3 |

| Sensor |  |
| --- | --- |
| Type | 1/1.8-inch CMOS |
| Pixels | 6.1MP |

| Video performance - Dynamic range |  |
| --- | --- |
| 5MP (4:3) mode | 97 dB WDR (97+16 dB with iAE) |
| 5MP (16:9) mode | 97 dB WDR (97+16 dB with iAE) |
| 1080p mode | 103 dB WDR (103+16 dB with iAE) |

| Video performance – Sensitivity (3200K, 89% reflectivity, 30% IRE, F1.2) |  |
| --- | --- |
| Color 5MP mode | 0.0121 lx |
| Color 1080p mode | 0.00825 lx |
| Mono 5MP mode | 0.004 lx |
| Mono 1080p mode | 0.00275 lx |

| Video streaming |  |
| --- | --- |
| Video compression | H.264 (ISO/IEC 14496-10); M- JPEG, JPEG |
| Streaming | Multiple configurable streams in H.264 and M-JPEG, configurable frame rate and bandwidth. Regions of Interest (ROI) |
| Overall IP Delay | Min. 120 ms, Max. 340 ms |
| GOP structure | IP, IBP, IBBP |
| Encoding interval | 1 to 30 fps |
| Encoder regions | Up to 8 areas with encoder quality settings per area |

| Video resolution |  |
| --- | --- |
| 5MP (16:9) | 2992 X 1680 |
| 5MP (4:3) | 2704 X 2032 |
| 1080p HD | 1920 X 1080 |
| 720p HD | 1280 x 720 |
| Upright 9:16 (cropped) | 400 x 720 |
| D1 4:3 (cropped) | 704 x 480 |
| 480p SD | Encoding: 704 x 480; Displayed: 854 x 480 |
| 432p SD | 768 x 432 |
| 288p SD | 512 x 288 |
| 240p SD | Encoding: 352 x 240; Displayed: 432 x 240 |
| 144p SD | 256 x 144 |

| Video functions |  |
| --- | --- |
| Day/Night | Color, Monochrome, Auto |
| Adjustable picture settings | Contrast, Saturation, Brightness |
| White Balance | 2500 to 10000K, 4 automatic modes (Standard, SON/SOX, Basic, Dominant color), Manual mode and Hold mode |
| Shutter | Automatic Electronic Shutter (AES) Fixed selectable Default shutter |
| Backlight compensation | Off, Auto, intelligentAE (iAE) |
| Noise reduction | intelligent Dynamic Noise Reduction (iDNR) with separate temporal and spatial adjustments |
| Contrast enhancement | On/off |
| Sharpness | Sharpness enhancement level selectable |
| Privacy Masking | Four independent areas, fully programmable |
| Video Motion Analysis | Intelligent Video Analysis (IVA) |
| User modes | 9 modes |
| Other functions | Image flip, Pixel counter, Video watermarking, Display stamping |

| Audio streaming |  |
| --- | --- |
| Standard | G.711, 8 kHz sampling rate L16, 16 kHz sampling rate AAC-LC, 48 kHz sampling rate AAC-LC, 80 kHz sampling rate |
| Signal-to-Noise Ratio | >50 dB |
| Audio Streaming | Full-duplex / half duplex |

| Input/output |  |
| --- | --- |
| Analog video out | SMB connector, CVBS (PAL/NTSC), 1 Vpp, 75 Ohm (service only) |
| Audio line in | 1 Vrms max, 18 kOhm typical, |
| Audio line out | 0.85 Vrms at 1.5 kOhm typical, |
| Audio connectors | 3.5 mm mono jack |
| Alarm input | 2 inputs |
| Alarm input activation | +5 VDC nominal; +40 VDC max. (DC-coupled with 50 kOhm pull-up resistor to +3.3 VDC) (< 0.5 V is low; > 1.4 V is high) |
| Alarm output | 1 output |
| Alarm output voltage | 30 VAC or +40 VDC max. Maximum 0.5 A continuous, 10VA (resistive load only) |
| Ethernet | RJ45 |
| Data port | RS-232/422/485 |

| Local storage |  |
| --- | --- |
| Internal RAM | 10 s pre-alarm recording |
| Memory card slot | Supports SDHC and SDXC microSD cards |
| Recording | Continuous recording, ring recording. alarm/events/schedule recording |

| Network |  |
| --- | --- |
| Protocols | IPv4, IPv6, UDP, TCP, HTTP, HTTPS, RTP/RTCP, IGMP V2/V3, ICMP, ICMPv6, RTSP, FTP, Telnet, ARP, DHCP, NTP (SNTP), 802.1x, DNS, DNSv6, DDNS (DynDNS.org, selfHOST.de, no-ip.com), SMTP, iSCSI, UPnP (SSDP), DiffServ (QoS), LLDP, SOAP, Dropbox, CHAP, digest authentication |
| Encryption | TLS 1.0, SSL, DES, 3DES, AES (optional) |
| Ethernet | 10/100 Base-T, auto-sensing, half/full duplex |
| Ethernet connector | RJ45 |
| Connectivity | ONVIF Profile S, Auto-MDIX |

| Software |  |
| --- | --- |
| Unit Configuration | Via web browser or Configuration Manager |
| Firmware update | Remotely programmable |
| Software viewer | Web browser, Bosch Video Client, or third party software |

| Optical |  |
| --- | --- |
| Lens mount | CS mount (C-mount with adapter ring) |
| Lens connector | Standard 4-pin DC-iris connector |
| Focus control | Motorized back-focus adjustment |
| Iris control | Automatic iris control |

| Mechanical |  |
| --- | --- |
| Dimensions (W x H x L) | 78 x 66 x140 mm (3.07 x 2.6 x 5.52 inch) without lens |
| Weight | 855 g (1.88 lb) without lens |
| Color | RAL 9006 Metallic Titanium |
| Tripod Mount | Bottom and top 1/4-inch 20 UNC |

| Environmental |  |
| --- | --- |
| Operating Temperature | -20°C to +50°C (-4°F to 122°F) |
| Storage Temperature | -30°C to +70°C (-22°F to +158°F) |
| Operating Humidity | 20% to 93% RH |
| Storage Humidity | up to 98% RH |
| Storage Humidity | up to 98% RH |

  - ACCESSORIES

    - Monitor cables

      - NBN-MCSMB-03M analog cable

      - NBN-MCSMB-30M analog cable

    - Software Options

      - MVC-FENC-AES BVIP AES 128 bit Encryption License

    - Power supplies

      - UPA-1220-50 power supply

      - UPA-1220-60 power supply

    - Lens

      - LVF-5005C-S4109 Varifocal SR Megapixel Lens

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

  - INSTALLATION

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to video system devices for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of camera settings to comply with specific customer’s need.

  - DEMONSTRATION

    - Demonstrate at final inspection that video management system and devices function properly.

END OF SECTION